Patrick Square Owners Association

Landscape RFP 2026 Fiscal Year

Landscape Maintenance – Common Areas 2026

Turf Maintenance- Common Areas

Mowing - (Common Areas) Once a week or as required to maintain a neat and attractive appearance April - September. Once every two weeks October - March.

Bermuda lawn to be cut at a height of 2-2 ½ inches. Fescue to be cut at a height of 3-3 ½ inches. Common areas include, the islands, main entrance way, roadsides from curb to sidewalk. All fertilization programs on the turf should be included in the contract price.

Edging - to be done with each cutting. All curbs and walkways to be mechanically edged. Plant beds to be defined to project a clear definition between turf and shrub location.

Trash and debris - to be removed from entire common areas

Blowing - all clippings and debris from sidewalks, curbs, parking lots, recreational areas during each visit.

Fertilization - Pre-emergence, fertilization and post emergence during growing season to control weed escapes.

1. First Application late January - early February- pre-emergent (split application)

Second Application late March - early April - fertilization application and (split application) pre-emergent application

3. Third Application May fertilization application

4. Fourth Application June - fertilization application

5. Fifth Application July - fertilization application

6. Sixth Application early September pre-emergent application

7. Post-Emergent should be used as often as need to control any weed escapes in the turf at no extra charge.

Leaf Removal – All leaves will be removed from all common areas every two weeks from October – December

Insecticides - to be applied to turf areas if needed and upon request for an extra charge.

Lake area - to be bi-weekly cut weekly during the growing season and as needed during the off season.

Wetlands - grass to be mowed to the ridge area weekly during the season and as needed during the off season, hill side to be mowed monthly.

Mulching and Flowers

Mulching

Spring (Feb - Mar) Mulching and Pine Straw price is not included in maintenance contract price. Mulch price should be per yard.  Pine straw should be per bale.

Mulching - Island on Thomas Green Blvd. Estimated for these areas is 30 cu. yds.

Mulching - Sign and Flower beds at entrances off Issaquena Trail. Estimated for these areas is 75 cu. yds.

Mulching - All tree rings. Estimated for these areas is 125 cu. yds.

Pine Straw - Planted tree beds on hill at entrance off Issaquena Trail. Estimated for these areas is 775 bales

Playground Mulch –

Total mulching/pine straw in the Spring. (Includes all tree rings at the front entrances at Issaquena Trail, the islands along Thomas Green Blvd, and playground)

Mulching - Lake house and Island on Keller Blvd. Estimated for these areas is

80 cu. yds.

Flowers

Flowers to be installed on both sides of the Issaquena Trail and Keller Blvd. entrance, Lake House, and Pavilion.  Seasonal annual flowers to be installed Spring and Fall.

Seasonal flowers are not included in maintenance contract price. Per square foot price:

Estimated for this property is 575 square feet.

Price to include purchase, installation, and fertilizers.

Shrub and Tree Maintenance

Pruning all ornamental shrubs and ground cover do be done twice a year.

Young sucker and sprouts to be removed as they appear

All leaves, branches and debris to be removed immediately after each pruning completion

Deep Tree Root Fertilization applications shall be applied upon request at an extra charge to all trees areas as needed for an additional cost per tree up to 5 inch caliper if additional trees are applied at the same time.

Shrub Fertilization at main entrance, and along Thomas Green Blvd., and at the lake house shall be applied upon request at an additional charge per treatment (5 treatments per year - March, April, June, August and November)

Street Tree Trimming shall be completed upon request at an extra charge as outlines below:

Pricing is as follows:

- Limbing up will require a minimum of 50 trees at a cost per tree.

- Tree Structural Pruning will require a minimum of 30 trees at a cost per tree.

Irrigation (Common areas)

A. Irrigation heads to be checked once a month for proper operation during the growing season.

B. Contractor will be responsible for the normal start up and shut down of the system.

C. All additional irrigation work will be completed at per hour plus materials.

Landscape Maintenance – Single Homes 2026

Turf Maintenance

Mowing – (Single-Family Homes- 220) Once a week or as required to maintain a neat and attractive appearance April - September. Twice per month October - November. Once per month December - March. Warm Season turf (Zoysia) to be cut at a height of 2-2 ½ inches. The landscape maintenance included on the Neighborhood Homes part of the contract is mowing, edging of the beds and concrete, and blowing off the concrete with each visit to the property. Leaves will be removed from all single-family units every 2 weeks (October to December).

C. Edging - to be done with each cutting. All curbs and walkways to be mechanically edged. Plant beds to be defined to project a clear definition between turf and shrub location.

D. Trash and debris - to be removed from entire common areas

E. Blowing - all clippings and debris from sidewalks and curbs during each visit.

| Item Description | Cost per Home | Total Cost |
| --- | --- | --- |
| Turf Maintenance – Single Family Homes |  |  |
| Notes: |  |  |

Landscape Maintenance – Townhomes 2026

Turf Maintenance

The landscape maintenance included on the Townhomes part of the contract is mowing, edging of the beds and concrete, blowing off the concrete with each visit to the property, trimming bushes, fertilizing grass (6 times per year), and maintaining the irrigation system. Leaves will be removed from all townhome units every 2 weeks (October to December).

Mowing – (Townhomes - 80) Once a week or as required to maintain a neat and attractive appearance April - September. Twice per month October - November. Once per month December - March. Warm Season turf (Zoysia, Bermuda, Centipede) to be cut at a height of 2-2 ½ inches.

Edging - to be done with each cutting. All curbs and walkways to be mechanically edged. Plant beds to be defined to project a clear definition between turf and shrub location.

Trash and debris - to be removed from entire common areas

Blowing - all clippings and debris from sidewalks and curbs during each visit.

Fertilization - Pre-emergence, fertilization and post emergence during growing season to control weed escapes.

- First Application late January - early February- pre-emergent (split application)

- Second Application late March - early April - fertilization application and (split application) pre-emergent application

- Third Application May fertilization application

- Fourth Application June - fertilization application

- Fifth Application July - fertilization application

- Sixth Application early September pre-emergent application

- Post-Emergent should be used as often as need to control any weed escapes in the turf at no extra charge.

Shrub and Tree Maintenance

Pruning all shrubs to be done every 6 weeks during growing season. Trees will be trimmed when necessary.

Irrigation heads to be checked once a month for proper operation during the growing season.

Contractor will be responsible for the normal start up and shut down of the irrigation system.

All additional irrigation work will be completed at per hour plus materials.

| Item Description | Cost per Home | Total Cost |
| --- | --- | --- |
| Turf, Shrubs, and Irrigation Maintenance – Townhomes |  |  |
| Notes: |  |  |

Landscape Maintenance – Villa Homes 2026

Turf Maintenance

The landscape maintenance included on the Villa Homes part of the contract is mowing, edging of the beds and concrete, and blowing off the concrete with each visit to the property, fertilizing grass (6 times per year), and maintaining the irrigation system. Leaves will be removed from all townhome units every 2 weeks (October to December).

Mowing – (Villas Homes - 26) Once a week or as required to maintain a neat and attractive appearance April - September. Twice per month October - November. Once per month December - March. Warm Season turf (Zoysia, Bermuda, Centipede) to be cut at a height of 2-2 ½ inches.

Edging - to be done with each cutting. All curbs and walkways to be mechanically edged. Plant beds to be defined to project a clear definition between turf and shrub location.

Trash and debris - to be removed from entire common areas

Blowing - all clippings and debris from sidewalks and curbs during each visit.

Fertilization - Pre-emergence, fertilization and post emergence during growing season to control weed escapes.

- First Application late January - early February- pre-emergent (split application)

- Second Application late March - early April - fertilization application and (split application) pre-emergent application

3. Third Application May - fertilization application

4. Fourth Application June - fertilization application

5. Fifth Application July - fertilization application

6. Sixth Application early September - pre-emergent application

7. Post-Emergent should be used as often as need to control any weed escapes in the turf at no extra charge.

Shrub and Tree Maintenance

Pruning all shrubs to be done every 6 weeks during growing season. Trees will be trimmed when necessary.

Irrigation heads to be checked once a month for proper operation during the growing season.

Contractor will be responsible for the normal start up and shut down of the irrigation system.

All additional irrigation work will be completed at per hour plus materials.

| Item Description | Cost per Home | Total Cost |
| --- | --- | --- |
| Turf, Shrubs, and Irrigation Maintenance – Villa Homes |  |  |
| Notes: |  |  |